- Dental handpiece cleaning, disinfection and sterilization operation guidelines

- Step one

- In accordance with the standard preventive measures, additional protective mask.

- Step two

- The chair, after the pre cleaning treatment timely brake pedal flushing lumen for 30 seconds, with a wet cotton ball or 75% alcohol cotton ball wipe the surface of the mobile phone timely removal of visible dirt into the recycling box temporary special.

- Step three

- Moisturizing preservation. Add a special multi-enzyme solution to recover a little moisturizer preservation

- Step four

- Recovery, according to different doctors were placed, sealed delivery to the disinfection chamber

- Step five

- Rinse

- Manual cleaning: with enzyme cleaning solution or neutral cleaning solution for handpiece surface scrubbing lavage in the flow of water rinsing → rinse water surface and lumen

- Full automatic cleaning machine cleaning, the handpiece is securely installed in a dedicated washing rack, automatic completion of cleaning, rinsing, disinfection process

- Step six

- Dry：

- 1.The manual cleaning with high-pressure air gun dry cavity and surface for handpiece

- 2.Automatic washing machine cleaning: automatic drying, disinfection

- Step seven

- Maintenance: check the cleanliness of the choice of mobile phone, mobile phone cleaning oiling machine with air pump to complete the cleaning maintenance procedure or manual filling, make it dry.

- Step eight

- Packing: use paper and plastic bags, sealing or packaging materials in line with packaging specification

- Step nine

- Sterilization: first vacuum high-pressure steam sterilization sterilization or sterilization bare cassette sterilizer

- Step ten

- Release: check the sterilization monitoring, the use of sterile containers loaded under the use of clinical departments